🌡️ EnviroTime – Digital Clock with Real-Time Temperature Monitoring

An Embedded C based real-time monitoring and digital clock system using the LPC2148 microcontroller.

📌 Project Overview

EnviroTime is a real-time embedded system developed using the LPC2148 ARM7 microcontroller. The system combines a digital clock with real-time ambient temperature monitoring.

The system uses an RTC (Real-Time Clock) to maintain and display the current time, date, day, month, and year. An LM35 temperature sensor is interfaced with the LPC2148's on-chip ADC to measure the ambient temperature.

The clock and temperature information are continuously displayed on a 16×2 LCD.

The system also provides:

- ⏰ Alarm functionality

- 🔐 Password-protected editing

- 🔢 4×4 matrix keypad input

- 🔘 Switch-based edit mode

- 🌡️ Real-time temperature monitoring

- 🔔 Buzzer indication

The project workflow and functionality are based on the project specification.

🎯 Aim

To design and develop a real-time embedded system using the LPC2148 microcontroller that displays accurate digital clock information along with continuous ambient temperature monitoring, while allowing secure user-controlled modification of time and alarm settings through switch-based polling and keypad authentication.

🎯 Objectives

- Implement RTC for accurate time, date, day, month, and year.

- Monitor ambient temperature using LM35 through the on-chip ADC.

- Display clock and temperature information on a 16×2 LCD.

- Provide user-defined alarm setting functionality.

- Interface a 4×4 matrix keypad for user input.

- Implement password-based authentication for editing.

- Use switch polling to enter edit mode.

- Generate buzzer alerts for alarms and system notifications.

🧩 Block Diagram

┌───────────────────┐

│      LPC2148      │

│  ARM7 Controller  │

└─────────┬─────────┘

│

┌─────────────────────┼─────────────────────┐

│                     │                     │

▼                     ▼                     ▼

┌────────────┐        ┌────────────┐        ┌────────────┐

│    RTC     │        │    ADC     │        │   Keypad   │

│ Time/Date  │        │            │        │    4×4     │

└────────────┘        └─────┬──────┘        └────────────┘

│

▼

┌────────────┐

│    LM35    │

│ Temperature│

│   Sensor   │

└────────────┘

LPC2148

│

┌────────┴────────┐

▼                 ▼

┌──────────────┐   ┌──────────────┐

│    16×2 LCD  │   │    Buzzer    │

│ Time + Temp  │   │ Alarm Alert  │

└──────────────┘   └──────────────┘

▲

│

┌──────────────┐

│    Switch    │

│  Edit Mode   │

└──────────────┘

The original project specification identifies the LPC2148 as the central controller and shows the RTC, ADC/LM35, keypad, switch, LCD, and buzzer as the major system blocks.

🔧 Hardware Requirements

| Component | Purpose |
| --- | --- |
| LPC2148 | Main ARM7 microcontroller |
| RTC | Maintains time and date |
| LM35 | Measures ambient temperature |
| 16×2 LCD | Displays time, date, and temperature |
| 4×4 Matrix Keypad | Password and user input |
| Switch | Enters edit/alarm control mode |
| Buzzer | Alarm and notification indication |
| USB-UART Converter / DB-9 Cable | Serial/programming interface |

These are the hardware components specified for the project.

💻 Software Requirements

- Embedded C

- Flash Magic

- LPC2148 development environment

The project uses Embedded-C programming and Flash Magic as specified in the project requirements.

⚙️ Main Modules

1. LPC2148 Microcontroller

The LPC2148 acts as the main controller of the entire system.

It coordinates:

- RTC

- LCD

- ADC

- LM35

- Keypad

- Switch

- Buzzer

2. RTC – Real-Time Clock

The RTC maintains the current:

- Hour

- Minute

- Second

- Day

- Date

- Month

- Year

The controller continuously reads the RTC values and displays them on the LCD.

Example:

HH:MM:SS

DD/MM/YYYY

3. LM35 Temperature Sensor

The LM35 is used to measure ambient temperature.

The sensor provides an analog voltage corresponding to temperature.

LM35

│

│ Analog Signal

▼

LPC2148 ADC

│

▼

Temperature Conversion

│

▼

16×2 LCD

The project specification states that LM35 is interfaced through the LPC2148's on-chip ADC and the measured temperature is displayed on the LCD.

🖥️ LCD Display

The 16×2 LCD provides the user interface.

A typical display format is:

20:12:35 T:28°C

20/04/2026 MON

The LCD continuously displays the current RTC information along with the measured temperature.

🔢 4×4 Matrix Keypad

The keypad is used for:

- Password entry

- User input

- Editing parameters

- Alarm setting

- Password change

After entering edit mode, the user can navigate through the available options using the keypad.

🔐 Password Authentication

One of the important features of EnviroTime is basic security for editing system parameters.

Authentication Flow

Edit Switch Pressed

│

▼

Password Request

│

▼

Enter Password

via Keypad

│

▼

Compare Password

/         \

YES          NO

│            │

▼            ▼

Allow Editing   Access Denied

│            │

▼            ▼

Edit Parameters   Wrong Attempt

Counter

The project requires password validation before allowing modification of system parameters. A wrong password results in access denial and increments the wrong-attempt counter.

🚨 Wrong Password Handling

The system includes a simple security mechanism.

If the password is entered incorrectly:

Wrong Password

│

▼

Access Denied

│

▼

Increment Counter

│

▼

3 Wrong Attempts?

/       \

YES       NO

│         │

▼         ▼

System     Continue

Locked     Normal Flow

After three incorrect attempts, the project specification states that edit mode can be locked temporarily or until reset, with "System Locked" displayed and an optional buzzer alert.

🔘 Switch-Based Edit Mode

The system continuously checks the switch status in the main loop.

Normal Monitoring

│

▼

Check Switch

│

┌───┴────┐

│        │

Not Pressed Pressed

│        │

▼        ▼

Continue   Enter

Monitoring Edit Mode

When the edit switch is pressed, the controller enters the editing process and requests password authentication.

⚙️ Editable Parameters

After successful password authentication, the user can modify:

1. RTC Settings

- Current time

- Day

- Date

2. Alarm Settings

- Alarm time

3. Password

- Password change option

These editable parameters are specified in the project document.

⏰ Alarm System

The system continuously monitors the configured alarm time.

When the current RTC time matches the alarm time:

RTC Time

│

▼

Compare with

Alarm Time

│

▼

Match?

│

YES

│

▼

Buzzer ON

│

▼

Press Alarm Switch

│

▼

Buzzer OFF

When the alarm time is reached, the buzzer provides an indication. The alarm can be stopped using the alarm switch.

🔄 System Working

The complete system operates in the following sequence:

Step 1 – Initialization

Initialize:

- RTC

- LCD

- Switch

- Keypad

- ADC

- Buzzer

Step 2 – Normal Monitoring

The controller enters continuous monitoring mode.

Step 3 – Read RTC

Read:

Hour

Minute

Second

Day

Date

Month

Year

Step 4 – Read Temperature

LM35 provides an analog signal.

LM35 → ADC → Temperature Value

Step 5 – LCD Display

Display time/date and temperature.

Step 6 – Alarm Monitoring

Compare current time with configured alarm time.

Step 7 – User Input

Check whether the edit switch has been pressed.

Step 8 – Authentication

If edit mode is requested:

Edit Switch

↓

Password

↓

Authentication

Step 9 – Parameter Editing

If authentication succeeds:

RTC Time

Alarm Time

Password

can be modified.

Step 10 – Return to Normal Mode

After editing:

Save Parameters

↓

Exit Edit Mode

↓

Normal Monitoring

The system then resumes RTC display, temperature monitoring, alarm monitoring, and buzzer indication.

🔄 Overall System Flowchart

START

│

▼

Initialize Peripherals

│

▼

┌──────────────────────┐

│   Read RTC Values    │

└──────────┬───────────┘

│

▼

┌──────────────────────┐

│ Read LM35 through ADC│

└──────────┬───────────┘

│

▼

Display on 16×2 LCD

│

▼

Check Alarm Time

│

┌────┴────┐

│         │

Match     No Match

│         │

▼         │

Buzzer ON      │

│         │

└────┬────┘

▼

Check Edit Switch

│

┌────┴────┐

│         │

Press    Not Pressed

│         │

▼         │

Request Password │

│         │

▼         │

Validate Password

/          \

Correct       Wrong

│             │

▼             ▼

Enter Edit Mode  Access Denied

│

▼

Edit RTC / Alarm /

Password

│

▼

Save Changes

│

▼

Return to Monitoring

│

└──────────────► Repeat

📁 Suggested Project Folder Structure

EnviroTime/

│

├── README.md

│

├── source/

│   ├── main.c

│   ├── rtc.c

│   ├── rtc.h

│   ├── lcd.c

│   ├── lcd.h

│   ├── keypad.c

│   ├── keypad.h

│   ├── adc.c

│   ├── adc.h

│   └── ...

│

├── include/

│   └── header_files/

│

├── docs/

│   ├── block_diagram.png

│   ├── flowchart.png

│   └── circuit_diagram.png

│

├── images/

│   ├── hardware_setup.jpg

│   ├── lcd_display.jpg

│   └── project_output.jpg

│

└── README.md

Adjust the filenames above to match the actual files in your project repository.

🛠️ Technologies Used

| Category | Technology |
| --- | --- |
| Microcontroller | LPC2148 |
| Processor | ARM7 |
| Programming Language | Embedded C |
| Clock | RTC |
| Temperature Sensor | LM35 |
| Analog Interface | On-chip ADC |
| Display | 16×2 LCD |
| Input | 4×4 Matrix Keypad |
| Control | Switch Polling |
| Alert | Buzzer |
| Programming Tool | Flash Magic |

⭐ Key Features

- ⏱️ Real-time digital clock

- 📅 Date, day, month and year display

- 🌡️ Real-time ambient temperature monitoring

- 🔢 4×4 matrix keypad interface

- 🔐 Password-based authentication

- ⚙️ Secure parameter editing

- ⏰ User-configurable alarm

- 🔔 Buzzer notification

- 🔘 Switch-based edit mode

- 🖥️ 16×2 LCD user interface

🏠 Applications

The project can be useful in environments where timekeeping and environmental monitoring are required, including:

- Homes

- Offices

- Laboratories

- Hospitals

- Industrial environments

These application areas are also identified in the project specification.

📚 Concepts Demonstrated

This project demonstrates practical knowledge of:

- ARM7 microcontroller programming

- Embedded C

- GPIO

- RTC interfacing

- ADC interfacing

- Temperature sensor interfacing

- LCD interfacing

- Matrix keypad interfacing

- Switch polling

- Buzzer control

- Password authentication

- Embedded system integration

👩‍💻 Project Role

Role: Embedded Systems Developer

Responsibilities

- Developed embedded C modules for the system.

- Worked with LPC2148 peripherals.

- Implemented RTC-based clock functionality.

- Interfaced LM35 with the ADC.

- Implemented LCD display functionality.

- Implemented keypad-based user input.

- Implemented switch-based edit mode.

- Integrated alarm and buzzer functionality.

- Implemented password-based access control.

🎓 Project Learning

Through this project, practical experience was gained in integrating multiple hardware peripherals with an ARM7 microcontroller and developing a complete embedded application combining timekeeping, temperature monitoring, user interaction, security, and alarm functionality.

📌 Project Status

Project: EnviroTime – Digital Clock with Real-Time Temperature Monitoring

Platform: LPC2148 ARM7

Language: Embedded C

Status: Embedded System Project

👤 Author

Usharani Chatragadda

Embedded Systems / Embedded C Enthusiast

Technical Skills

C • C++ • Embedded C • Linux • ARM • TCP/IP

📜 License

This project is intended for educational and demonstration purposes.